# NETWORK INFRASTRUCTURE PRACTICAL MANUAL

Module: Network Infrastructure
Programme: Information Technology / Networking Support
Practical Assessments: 9
Total Marks: 100
Lab Size: 10 Student Workstations

# 1. MODULE OVERVIEW

This practical module equips students with the knowledge and skills required to design, install, configure, manage, test, and troubleshoot network infrastructure in a modern enterprise environment.

The practical exercises progress from basic cabling and hardware installation to Cisco router and switch configuration, VLAN implementation, network services, and troubleshooting.

# 2. AVAILABLE EQUIPMENT

## Structured Cabling Equipment

| Item | Quantity |
| --- | --- |
| Crimping Tools | 4 |
| Punchdown Tools | 4 |
| Patch Panels | 4 |
| Network Patch Boxes | 13 |
| 0.5m Fly Leads | 13 |
| 1m Fly Leads | 13 |
| Network Cable (500m) | 4 Rolls |

## Cisco Equipment

| Item | Quantity |
| --- | --- |
| Cisco Console Cables | 4 |
| Cisco 1941 Routers | 2 |
| Cisco ISR 4331 Routers | 2 |
| Cisco Catalyst 3560 Switches | 2 |

## Network Infrastructure Equipment

| Item | Quantity |
| --- | --- |
| 24-Port Switches | 4 |
| 24-Port Patch Panels | 4 |
| 4U Switch Cabinets | 4 |

## Computer Equipment

| Item | Quantity |
| --- | --- |
| Lenovo Desktop PCs | 10 |
| LG Screens | 10 |
| Keyboard & Mouse Sets | 10 |

## Printing Equipment

| Item | Quantity |
| --- | --- |
| HP LaserJet 600 M602 Printer | 1 |
| HP Color LaserJet 5500DN Duplex Printer | 1 |
| HP Colour LaserJet Professional CP5225N Printer | 1 |

## Toolkits

| Item | Quantity |
| --- | --- |
| SPROTEK 58-Piece Computer Technician Toolkit | 2 |
| SPROTEK 10-Piece Network Installation Toolkit | 2 |

## Classroom Equipment

| Item | Quantity |
| --- | --- |
| Projector | 1 |
| Projector Mounting | 1 |
| Whiteboard | 1 |

# 3. LEARNING OUTCOMES

Upon successful completion of this module, students will be able to:

- Safely use networking tools and equipment.

- Construct Ethernet cables using industry standards.

- Install and terminate patch boxes and patch panels.

- Build structured cabling systems.

- Install and manage switch cabinets.

- Access and configure Cisco IOS devices.

- Configure Cisco switches and routers.

- Implement VLANs and network segmentation.

- Configure shared network resources.

- Troubleshoot enterprise network faults.

- Document network installations professionally.

# PRACTICAL 1: NETWORK TOOLS AND SAFETY

## Objective

Identify networking tools and apply safe working procedures.

## Activities

- Identify networking tools.

- Explain the use of each tool.

- Demonstrate safe handling.

- Complete a tool inventory sheet.

- Apply health and safety procedures.

## Deliverable

Completed Tool Identification Worksheet.

## Assessment Rubric (10 Marks)

| Criteria | Marks |
| --- | --- |
| Tool identification | 4 |
| Understanding tool functions | 2 |
| Safe tool usage | 2 |
| Worksheet completion | 2 |
| Total | 10 |

# PRACTICAL 2: ETHERNET CABLE CONSTRUCTION

## Objective

Construct and test Ethernet patch cables using T568B.

## Activities

- Measure and cut cable.

- Strip cable jacket.

- Arrange conductors according to T568B.

- Crimp RJ45 connectors.

- Test completed cables.

### T568B Wiring Standard

- White/Orange

- Orange

- White/Green

- Blue

- White/Blue

- Green

- White/Brown

- Brown

## Deliverable

Three tested Ethernet cables.

## Assessment Rubric (10 Marks)

| Criteria | Marks |
| --- | --- |
| Correct cable sequence | 4 |
| Cable preparation | 2 |
| Crimp quality | 2 |
| Connectivity testing | 2 |
| Total | 10 |

# PRACTICAL 3: PATCH BOX INSTALLATION

## Objective

Install and terminate network outlets.

## Activities

- Install patch boxes.

- Terminate using T568B.

- Label outlets.

- Test connectivity.

## Deliverable

Fully functional network outlet.

## Assessment Rubric (10 Marks)

| Criteria | Marks |
| --- | --- |
| Correct termination | 4 |
| Use of punchdown tool | 2 |
| Cable management | 2 |
| Labelling | 2 |
| Total | 10 |

# PRACTICAL 4: PATCH PANEL INSTALLATION

## Objective

Install and document a patch panel.

## Activities

- Mount patch panel.

- Terminate network runs.

- Label ports.

- Create port schedule.

- Test all ports.

## Deliverable

Completed patch panel installation.

## Assessment Rubric (10 Marks)

| Criteria | Marks |
| --- | --- |
| Correct patch panel termination | 4 |
| Port numbering | 2 |
| Documentation | 2 |
| Testing verification | 2 |
| Total | 10 |

# PRACTICAL 5: CISCO DEVICE ACCESS AND BASIC CONFIGURATION

## Objective

Access Cisco IOS and perform basic device configuration.

## Equipment

- Console Cables

- Cisco 1941 Router

- Cisco ISR4331 Router

- Cisco Catalyst 3560 Switch

## Activities

- Access Cisco devices using console cables.

- Configure hostname.

- Configure passwords.

- Configure login banner.

- Save configurations.

### Example Configuration

1 enable 2 configure terminal 3 4 hostname POTCH-R1 5 6 enable secret Cisco123 7 8 line console 0 9 password Cisco123 10 login 11 12 banner motd # Unauthorized Access Prohibited # 13 14 end 15 copy running-config startup-config

## Deliverable

Configuration screenshots and configuration file.

## Assessment Rubric (10 Marks)

| Criteria | Marks |
| --- | --- |
| Console connectivity | 2 |
| IOS navigation | 3 |
| Device configuration | 3 |
| Save and verify configuration | 2 |
| Total | 10 |

# PRACTICAL 6: LAN CONSTRUCTION AND SWITCH CONFIGURATION

## Objective

Build a LAN using a Cisco Catalyst switch.

## Equipment

- 10 PCs

- Cisco Catalyst 3560 Switch

- Patch Panel

- Fly Leads

## Activities

- Connect all 10 PCs.

- Configure static IP addresses.

- Configure switch management interface.

- Verify connectivity.

- Create a network diagram.

### IP Addressing Scheme

| Device | IP Address |
| --- | --- |
| PC1 | 192.168.10.1 |
| PC2 | 192.168.10.2 |
| PC3 | 192.168.10.3 |
| PC4 | 192.168.10.4 |
| PC5 | 192.168.10.5 |
| PC6 | 192.168.10.6 |
| PC7 | 192.168.10.7 |
| PC8 | 192.168.10.8 |
| PC9 | 192.168.10.9 |
| PC10 | 192.168.10.10 |
| Switch VLAN 1 | 192.168.10.254 |

Subnet Mask: 255.255.255.0

### Verification Commands

1 show running-config 2 show vlan brief 3 show mac address-table

## Deliverable

Operational LAN and topology diagram.

## Assessment Rubric (15 Marks)

| Criteria | Marks |
| --- | --- |
| Physical installation | 3 |
| Switch configuration | 4 |
| IP configuration | 4 |
| Connectivity testing | 4 |
| Total | 15 |

# PRACTICAL 7: ROUTER CONFIGURATION AND INTER-NETWORK COMMUNICATION

## Objective

Configure routers and establish communication between networks.

## Equipment

- Cisco 1941 Router

- Cisco ISR4331 Router

- Cisco Switches

- PCs

## Activities

Create two networks:

### Network A

1 192.168.10.0/24

### Network B

1 192.168.20.0/24

Configure router interfaces.

### Example Configuration

1 interface g0/0 2 ip address 192.168.10.1 255.255.255.0 3 no shutdown 4 5 interface g0/1 6 ip address 192.168.20.1 255.255.255.0 7 no shutdown

### Verification Commands

1 show ip interface brief 2 ping 3 tracert

## Deliverable

Successful communication between both networks.

## Assessment Rubric (10 Marks)

| Criteria | Marks |
| --- | --- |
| Router configuration | 4 |
| Interface addressing | 2 |
| Connectivity testing | 2 |
| Documentation | 2 |
| Total | 10 |

# PRACTICAL 8: VLAN IMPLEMENTATION AND NETWORK SEGMENTATION

## Objective

Create VLANs and segment network traffic.

## Equipment

- Cisco Catalyst 3560 Switches

- 10 PCs

## VLAN Design

| VLAN | Department | Devices |
| --- | --- | --- |
| VLAN 10 | Administration | PC1-PC3 |
| VLAN 20 | Lecturers | PC4-PC6 |
| VLAN 30 | Students | PC7-PC10 |

## Activities

- Create VLANs.

- Assign ports to VLANs.

- Verify VLAN membership.

- Test communication.

- Document VLAN design.

### Example Commands

1 vlan 10 2 name ADMIN 3 4 vlan 20 5 name LECTURERS 6 7 vlan 30 8 name STUDENTS

### Verification

1 show vlan brief

## Deliverable

Working segmented network.

## Assessment Rubric (15 Marks)

| Criteria | Marks |
| --- | --- |
| VLAN creation | 4 |
| Port assignment | 4 |
| Connectivity verification | 4 |
| Documentation | 3 |
| Total | 15 |

# PRACTICAL 9: ENTERPRISE NETWORK DEPLOYMENT AND TROUBLESHOOTING

## Objective

Deploy and troubleshoot a complete Cisco-based enterprise network.

## Equipment

- Cisco Routers

- Cisco Switches

- Switch Cabinets

- Patch Panels

- Printers

- 10 PCs

## Activities

- Install switch cabinet.

- Install patch panels.

- Configure switches.

- Configure routers.

- Implement VLANs.

- Configure printers.

- Test and troubleshoot the network.

### Introduced Faults

- Incorrect VLAN assignment

- Incorrect subnet mask

- Incorrect default gateway

- Duplicate IP address

- Faulty patch lead

- Disabled switch port

- Router interface shutdown

- Missing route

### Troubleshooting Commands

1 show running-config 2 show vlan brief 3 show interfaces 4 show ip route 5 show ip interface brief 6 show mac address-table 7 ping 8 tracert

## Deliverable

Completed troubleshooting report.

## Assessment Rubric (10 Marks)

| Criteria | Marks |
| --- | --- |
| Fault identification | 3 |
| Troubleshooting methodology | 3 |
| Correct resolution | 2 |
| Professional report | 2 |
| Total | 10 |

# PRACTICAL OBSERVATION CHECKLIST

| Competency | Yes | No |
| --- | --- | --- |
| Uses tools safely | ☐ | ☐ |
| Uses crimping tools correctly | ☐ | ☐ |
| Uses punchdown tools correctly | ☐ | ☐ |
| Applies T568B correctly | ☐ | ☐ |
| Configures Cisco devices correctly | ☐ | ☐ |
| Demonstrates troubleshooting skills | ☐ | ☐ |
| Maintains clean workstation | ☐ | ☐ |
| Demonstrates teamwork | ☐ | ☐ |
| Produces accurate documentation | ☐ | ☐ |

# FINAL CAPSTONE PROJECT

## Scenario

Potchefstroom Campus requires a fully operational networking laboratory for 10 workstations. Student teams must design, install, configure, test, secure, document, and troubleshoot the complete network infrastructure.

## Infrastructure Requirements

### End User Devices

- 10 Desktop PCs

- 10 Monitors

- 10 Keyboard and Mouse Sets

### Cisco Equipment

- 2 Cisco Catalyst 3560 Switches

- 2 Cisco 1941 Routers

- 2 Cisco ISR 4331 Routers

- 4 Cisco Console Cables

### Infrastructure Equipment

- 4 Patch Panels

- 4 Switch Cabinets

- 13 Network Outlets

- Structured Cabling System

- Fly Leads

### Shared Resources

- 3 Network Printers

- Projector

- Whiteboard

## Project Tasks

### Phase 1: Planning

- Produce physical topology

- Produce logical topology

- Create IP addressing plan

- Design VLAN structure

### Phase 2: Infrastructure Installation

- Install switch cabinets

- Install patch panels

- Terminate outlets

- Test cabling

### Phase 3: Cisco Switch Configuration

- Configure hostnames

- Configure passwords

- Configure management interfaces

- Configure VLANs

### Phase 4: Cisco Router Configuration

- Configure interfaces

- Configure routing

- Verify routing tables

- Test communication

### Phase 5: Shared Resource Configuration

- Configure network printers

- Verify access from all VLANs

### Phase 6: Testing and Troubleshooting

- Verify connectivity

- Verify VLAN functionality

- Verify routing

- Resolve instructor-introduced faults

### Phase 7: Documentation

Submit:

- Network topology diagram

- IP addressing table

- VLAN allocation table

- Switch configurations

- Router configurations

- Connectivity test results

- Troubleshooting report

# CAPSTONE PROJECT RUBRIC (20 MARKS)

| Assessment Area | Marks |
| --- | --- |
| Network Design and Planning | 4 |
| Structured Cabling Installation | 3 |
| Patch Panel and Cabinet Configuration | 2 |
| Cisco Switch Configuration | 3 |
| Cisco Router Configuration | 3 |
| VLAN Configuration | 2 |
| Testing and Troubleshooting | 2 |
| Documentation and Professionalism | 1 |
| Total | 20 |

# ASSESSMENT SUMMARY

| Assessment | Marks |
| --- | --- |
| Practical 1 | 10 |
| Practical 2 | 10 |
| Practical 3 | 10 |
| Practical 4 | 10 |
| Practical 5 | 10 |
| Practical 6 | 15 |
| Practical 7 | 10 |
| Practical 8 | 15 |
| Practical 9 | 10 |
| Total Continuous Assessment | 100 |

# ACHIEVEMENT LEVELS

| Percentage | Achievement |
| --- | --- |
| 80 - 100% | Excellent |
| 70 - 79% | Competent |
| 60 - 69% | Satisfactory |
| 50 - 59% | Basic Competence |
| Below 50% | Not Yet Competent |

## Expected Graduate Competencies

Students who successfully complete this practical programme will be able to:

- Install structured cabling systems.

- Configure patch panels and switch cabinets.

- Configure Cisco Catalyst switches.

- Configure Cisco 1941 and ISR4331 routers.

- Implement VLANs and network segmentation.

- Configure network printers and shared resources.

- Troubleshoot physical and logical network issues.

- Deploy and document a small enterprise network consisting of 10 workstations using industry-standard networking practices.